2]

#include <stdio.h>

#define SIZE 5

int cqueue[SIZE], front = -1, rear = -1;

int isFull() {

return (front == 0 && rear == SIZE - 1) || (rear + 1 == front); }

int isEmpty() {

return front == -1; }

void enqueue(int val) {

if (isFull())

printf("Queue is Full!\n");

else {

if (front == -1) front = 0;

rear = (rear + 1) % SIZE;

cqueue[rear] = val;

printf("%d inserted.\n", val); }}

void dequeue() {

if (isEmpty())

printf("Queue is Empty!\n");

else {

printf("Deleted: %d\n", cqueue[front]);

if (front == rear) front = rear = -1;

else front = (front + 1) % SIZE;  }}

void display() {

if (isEmpty())

printf("Queue is Empty!\n");

else {

printf("Queue: ");

int i = front;

while (1) {

printf("%d ", cqueue[i]);

if (i == rear) break;

i = (i + 1) % SIZE; }

printf("\n");}}

void peek() {

if (isEmpty())

printf("Queue is Empty!\n");

else

printf("Front Element: %d\n", cqueue[front]);}

int main() {

int ch, val;

do {

printf("\n1.Enqueue 2.Dequeue 3.Display 4.Peek 5.Exit\n");

scanf("%d", &ch);

switch (ch) {

case 1: printf("Enter value: "); scanf("%d", &val); enqueue(val); break;

case 2: dequeue(); break;

case 3: display(); break;

case 4: peek(); break; }

} while (ch != 5);

return 0;}